# 🩵 LEVEL 2 — THE SKILLED RIDER

## SECTION 10 — ADVANCED TERRAIN MASTERY & RECOVERY SCENARIOS

(TTN Beginner-Plus Edition — “Calm Under Pressure, Smart on the Line”)

### 🧠 Step 1 — The TTN Mindset of Mastery

Control is what keeps you safe.
Composure is what makes you skilled.

💬 TTN Truth: The goal isn’t to conquer terrain — it’s to work with it.

When a trail gets tough, slow down, breathe, and make deliberate movements.
The machine can do far more than most drivers realize — if you stay calm.

# ⛰️ Step 2 — Advanced Hill Climbing

Hill climbs test power, traction, and judgment all at once.
A confident rider learns to read the hill before even touching the throttle.

### 📍 Reading the Hill

1️⃣ Stand 20 ft back. Look for tire ruts — deep = soft terrain.
2️⃣ Spot your “bail-out zone” — a flat or safe area to stop if needed.
3️⃣ Mark your exit line before entering the incline.

💡 TTN Tip: Never start a climb unsure of where it ends.

### ⚙️ Climbing Technique

1️⃣ Engage Low Gear (L) and 4WD or AWD.
2️⃣ Roll into throttle smoothly; keep steady pressure — don’t “blip.”
3️⃣ Keep wheels straight — side angle = rollover risk.
4️⃣ Maintain momentum but avoid speed.
5️⃣ If you lose traction → brake gently, shift to Reverse, and roll back straight down.

⚠️ Never spin tires — that’s how you dig ruts and lose grip.

💬 TTN Truth: Momentum is not speed. It’s control in motion.

### 🧰 Garage Lesson #42 — “Hill Recovery Drill”

1️⃣ Find a mild incline with good traction.
2️⃣ Drive halfway up, stop smoothly, hold brake.
3️⃣ Shift to Reverse, release brake slowly, let gravity roll you down straight.
4️⃣ Repeat until it feels automatic.

🧠 Goal: Eliminate panic reaction when losing traction mid-hill.

# ↔️ Step 3 — Off-Camber & Sidehill Traversing

Off-camber means the trail tilts sideways — one side higher than the other.
This is where most new riders freeze or over-correct.

### ⚙️ Technique

- Drive slow and steady — momentum increases tip risk.

- Keep uphill tires on the high line.

- Keep wheels pointed slightly uphill for stability.

- Lean your body and passenger uphill.

- If machine slides sideways, steer gently downhill until stable.

💡 TTN Tip: If your inner ear feels off balance, your machine probably is too.

### ⚠️ Common Mistakes

- Turning downhill too sharply → roll risk.

- Stopping abruptly mid-camber → weight shifts wrong.

- Jerking throttle → tires dig instead of grip.

💬 TTN Truth: Gravity always wins — learn to guide it, not fight it.

### 🧰 Garage Lesson #43 — “Balance Line”

1️⃣ Mark a gentle side slope with cones.
2️⃣ Drive across at walking speed, no brake, steady throttle.
3️⃣ Passenger leans uphill; driver keeps light uphill steering.
4️⃣ Stop halfway, restart safely.

🧠 Why: Practicing sidehill balance on mild ground builds instinct before it matters on real terrain.

# 💦 Step 4 — Deep Water & Creek Crossings

Water can be friend or foe — it hides depth, rocks, and traction loss.

### 📍 Scouting Steps

1️⃣ Walk it first if under 2 ft deep — never blind-enter.
2️⃣ Look for entry/exit slopes that aren’t too steep.
3️⃣ Watch flow direction — moving current pushes harder than you think.
4️⃣ Identify submerged rocks by ripple patterns.

### ⚙️ Technique

- Switch to Low Gear and 4WD.

- Maintain steady throttle — don’t stop midstream.

- Keep a small bow wave in front of tires.

- Avoid wheel spin — slow and even.

- Once out, lightly apply brakes for 10 seconds to dry rotors.

💡 TTN Tip: If you splash belt intake or exhaust → stop immediately, open cover, and dry before continuing.

### ⚠️ Post-Water Check

| Component | What to Check | Why |
| --- | --- | --- |
| Belt Box | Moisture or steam | Prevent belt slip |
| Diff & Trans Fluids | Milky = water contamination | Replace within 24 hrs |
| Air Filter | Damp or muddy | Replace or dry before restart |

💬 TTN Truth: Water and electricity don’t negotiate — check before you ride again.

### 🧰 Garage Lesson #44 — “Controlled Crossing”

1️⃣ Create a mock “creek” using a shallow ditch or low puddle.
2️⃣ Practice entering slow, steady throttle, no splash.
3️⃣ Passenger watches wheel path.
4️⃣ Exit, stop, and inspect belt cover for leaks.

🧠 Goal: Develop calm crossing control, not speed-based reactions.

# 🪨 Step 5 — Technical Obstacles & Rock Gardens

These are the confidence tests of off-roading — ruts, boulders, logs, and uneven surfaces.
Technique and precision matter more than horsepower.

### ⚙️ The “Turtle Rule”

Slow enough to watch tires touch every rock — but steady enough not to lose momentum.

### 📍 Approach & Execution

1️⃣ Stop and study the obstacle.
2️⃣ Plan line — visualize front tires climbing, not dragging.
3️⃣ Keep wheels straight.
4️⃣ Engage Low Gear and 4WD Lock if equipped.
5️⃣ Apply smooth throttle; let tires crawl up and over.

💡 TTN Tip: Crawl until you hear minimal tire slip — that’s traction sweet spot.

### ⚠️ Common Mistakes

- Rushing → bouncing and bottom-outs.

- Spinning tires → polishing rocks, no grip.

- Oversteering → hits diff or A-arms on ledges.

💬 TTN Truth: Slow climbs make loud cheers. Fast ones make loud crunches.

### 🧰 Garage Lesson #45 — “Rock Crawl Drill”

1️⃣ Place 4x4 boards or bricks in staggered pattern.
2️⃣ Crawl over in Low gear, minimal throttle.
3️⃣ Passenger watches tire placement and calls “touch” when tire meets each block.
4️⃣ Repeat until driver can place tires by feel alone.

🧠 Why: Builds visual-to-muscle coordination — your eyes learn to predict tire behavior.

# 🪢 Step 6 — Winching Angles & Anchoring

A winch is only as safe as its setup.
You’re moving thousands of pounds of tension — treat it with respect.

💬 TTN Truth: Patience and precision move more weight than horsepower.

### ⚙️ Winching Setup 101

1️⃣ Park machine and set brake.
2️⃣ Put transmission in Neutral.
3️⃣ Unspool winch line 8–10 ft minimum before tensioning.
4️⃣ Use tree-saver strap around anchor (never wrap the rope itself).
5️⃣ Attach winch hook to strap with shackle facing outward.
6️⃣ Lay a dampener blanket (or jacket) midway on line to absorb recoil.
7️⃣ Stand clear — no one in front or behind the cable path.
8️⃣ Use slow pulls: 5–10 seconds on, 10 seconds off (prevents winch overheating).

💡 TTN Tip: Always anchor low — high angles lift machines instead of pulling forward.

### 🧭 Angle Awareness

| Scenario | Best Anchor | Pull Strategy |
| --- | --- | --- |
| Straight stuck | Tree directly in line | Single-line pull |
| Off-angle stuck | Tree slightly to side | Use pulley snatch block to redirect |
| Deep bog | Two trees / double line | Doubled rope halves strain on winch |

🧠 Why: Mechanical advantage + correct angle = less stress on every part.

# 🚜 Step 7 — Tandem Tow & Strap Recovery

Sometimes another machine is your best recovery tool.

### ⚙️ Strap Recovery Basics

1️⃣ Use rated recovery rope (minimum 2x machine weight).
2️⃣ Connect only to frame or tow point — never cage or suspension arms.
3️⃣ Clear bystanders before tension.
4️⃣ Pull in short bursts — no jerking.
5️⃣ Once free, stop and inspect both machines.

💬 TTN Truth: Smooth pulls save straps — snatched ones snap metal.

### 🧰 Garage Lesson #46 — “Tandem Recovery Practice”

1️⃣ In open space, bury one tire in soft dirt.
2️⃣ Second machine attaches strap to frame loop.
3️⃣ Execute gentle pull while stuck driver lightly throttles.
4️⃣ Reset and switch roles.

💡 TTN Tip: Always keep hand signals simple — 👋 “Stop”, ✊ “Ready”, 👍 “Good pull”.

# 💀 Step 8 — Rollover Response & Re-Righting

Even good drivers tip — terrain surprises everyone.
The difference between a mistake and a disaster is calm recovery.

### 🧍 What To Do Immediately

1️⃣ Check everyone’s okay — no movement until confirmed.
2️⃣ Shut off ignition and battery disconnect if accessible.
3️⃣ Fuel or oil leaking? → Step 10 ft back, keep fire extinguisher ready.
4️⃣ Remove riders from high side slowly — never crawl under frame.

💡 TTN Tip: Rollovers happen in seconds — safety recovery takes minutes. Take your time.

### ⚙️ Re-Righting Steps

1️⃣ Anchor winch or strap to roll cage base (not roof bar).
2️⃣ Use a second machine or tree as base anchor.
3️⃣ Pull slowly until machine pivots back upright.
4️⃣ Once upright, wait 5–10 minutes before restarting to allow fluids to settle.
5️⃣ Check oil, air filter, and fuel lines before running engine.

💬 TTN Truth: If it’s on its side more than a minute, change the oil when back at camp — guaranteed contamination.

### 🧰 Garage Lesson #47 — “Controlled Tip & Right Drill”

1️⃣ Lay old ATV or lawn frame on its side (not a real SxS).
2️⃣ Practice anchor setup and safe rope path.
3️⃣ Use 50% power on winch and guide frame back upright slowly.
4️⃣ Discuss line-of-fire safety with everyone.

🧠 Goal: Build calm reaction muscle memory for real rollovers.

# 🌫️ Step 9 — Mud & Deep Bog Extraction

Mud is fun until it’s 2 ft deep and the suction turns your machine into a statue.
Escaping it is more physics than power.

### ⚙️ Strategy

1️⃣ Stop spinning tires — dig deeper = worse.
2️⃣ Assess: how deep are tires and diffs?
3️⃣ Use recovery board or log under tires for traction.
4️⃣ Lightly throttle forward and backward to create wiggle room.
5️⃣ If no progress → use winch or tandem strap.
6️⃣ Clear mud from radiator ASAP — overheating happens quick.

💡 TTN Tip: Carry 2 short recovery boards — they double as ramps and table seats later.

### 🧰 Garage Lesson #48 — “Bog Escape Simulation”

1️⃣ Use sand pit or shallow ditch.
2️⃣ Drive in slowly until rear tires lose traction.
3️⃣ Practice recovery board placement and throttle modulation.
4️⃣ Time how long it takes to exit cleanly.

🧠 Why: Real mud teaches patience, not panic.

# 🧭 Step 10 — TTN Recovery Framework

When you’re stuck, ask yourself:

1️⃣ Stop — is everyone safe?
2️⃣ Assess — what’s actually stuck (tires, diff, or frame)?
3️⃣ Stabilize — no spinning, no digging deeper.
4️⃣ Select — winch, strap, dig, or board?
5️⃣ Execute — one method at a time, slow and steady.

💬 TTN Truth: Good recovery is quiet. If it’s loud, something’s about to break.

# 🧾 TTN Recovery Strategy Card (Printable)

STUCK CHECKLIST
✅ Engine off, assess safety.
✅ Identify stuck points.
✅ Plan anchor or pull line.
✅ Clear spectators.
✅ Communicate signals (👊 ready / 🖐 stop).
✅ Pull slow — watch angle and tire movement.
✅ After recovery, check:
 • Fluids
 • Radiator
 • Axles
 • Belt Cover

EMERGENCY BACKUP PLAN
📡 Channel 16 → “Trail Captain, recovery in progress, Trail 12 South.”
🧭 Drop GPS pin for location.
📷 Take photos if structural damage for later repair report.

💡 TTN Tip: A smart rider logs every recovery — it becomes your best teacher later.