# 🩵 LEVEL 1 — THE NEW RIDER

## SECTION 2 — CONTROLS, GAUGES & BASIC DRIVING

(TTN Garage-Class Edition — “Confidence Starts Before the Engine Does”)

### 🧠 Step 1 — Get Comfortable in the Driver’s Seat

Before you start the engine, sit down, breathe, and look around.
This isn’t just a seat — it’s your new control center. You’ll spend hours here, so make it fit you, not the other way around.

💬 TTN Truth: A confident driver isn’t the one who looks coolest — it’s the one who feels relaxed and in control before they ever hit the throttle.

#### 🪑 Seat Position

- Slide forward until you can fully press the brake pedal without stretching.

- Knees should stay slightly bent when you brake.

- You should never have to lean forward to reach anything.

💡 TTN Tip: You should be able to press the brake firmly while keeping your shoulders against the seat — that’s your “fit zone.”

#### 🔗 Harness or Seatbelt

Your harness is your body’s helmet. Buckle up every single time, even when moving 10 feet.

3-Point Seatbelt:

- Lap strap sits low across hips.

- Shoulder belt crosses between neck and shoulder.

4- or 5-Point Harness:
1️⃣ Buckle lap straps first.
2️⃣ Pull shoulder belts snug.
3️⃣ Connect center buckle last.

💬 TTN Truth: Harnesses don’t slow you down — they let you ride harder with confidence.

#### 🪞 Mirrors & Visibility

Adjust side mirrors to see your rear tires and a sliver of your machine.
Rear mirror should show your flag or whip if equipped.

💡 TTN Tip: If you can see your back tires in both side mirrors, you’ll always know where your width ends — a lifesaver on tight trails.

#### 🎛️ Switch Panel Familiarization

Take a minute to memorize your switches.
Label them temporarily with tape if needed until muscle memory kicks in.

Common switches include:

- Headlights / Light bar

- Whip lights

- Winch In/Out

- Fan override

- Horn

🧰 Garage Lesson #4 — Switch Memory Drill
1️⃣ Turn off all lights.
2️⃣ In the dark, try to find each switch by touch.
3️⃣ Practice flipping each one until you can do it without looking.
🧠 Goal: Your hands should know your machine even when your eyes can’t see it.

### ⚙️ Step 2 — Pedals, Gears & How It Moves

SxS controls feel like a car — but they respond faster and rougher.
Learning to blend throttle and brake smoothly will make or break your confidence.

#### 🦶 Pedals

- Right Pedal: Throttle (gas).

- Left Pedal: Brake.
There’s no clutch because your SxS uses a CVT (Continuously Variable Transmission).

💡 TTN Tip: Use one foot for both pedals. Resting your left foot on the brake burns belts and eats clutches.

#### 🧭 Gear Shifter (P–R–N–H–L)

| Gear | Use For | Notes |
| --- | --- | --- |
| P (Park) | Starting, parking, idling | Locks transmission |
| R (Reverse) | Backing up | Look over shoulder, not just mirrors |
| N (Neutral) | Winching, warming up | Free spin, no drive |
| H (High) | General trail riding | Smoother, faster |
| L (Low) | Steep climbs, mud, towing | Adds torque, lowers speed |

💬 TTN Truth: If it doesn’t slide into gear easily, tap the brake and wait one second. Forcing it teaches your transmission bad manners.

#### 🧰 Garage Lesson #5 — “Smooth Shift Drill”

1️⃣ Machine off, foot on brake.
2️⃣ Move through each gear — Park → Reverse → Neutral → High → Low — slowly.
3️⃣ Feel the slight click between each position.
4️⃣ Practice this five times before starting the engine.

🧠 Goal: Train your muscle memory to find gears gently. A smooth hand means longer transmission life.

### 🔑 Step 3 — Starting Up Like a Pro

1️⃣ Check that you’re in Park (P).
2️⃣ Foot on brake.
3️⃣ Buckle seatbelt — some rigs won’t rev unless you do.
4️⃣ Turn the key or press start.
5️⃣ Let it idle for a full minute, especially in cold weather.

💡 TTN Tip: Use the first 60 seconds to listen. Each sound tells you something — a rattle, hum, or whir becomes familiar over time.

💬 TTN Truth: Your machine will talk to you if you give it the silence to do so.

### 📊 Step 4 — Learn Your Dash & Gauges

Your dash is the conversation screen between you and your machine.
Don’t ignore it — it’s your most important communication tool.

Common Indicators:

- Speed (MPH) — how fast you’re actually moving.

- RPM (Revolutions per Minute) — how hard your engine’s working.

- Temp Gauge — normal range usually 170–205°F.

- Battery Voltage — should read around 12.5–14.4V when running.

- Hour Meter — tracks maintenance schedule.

Warning Lights:
⚠️ Check Engine: Read the code, don’t guess.
🧊 Temp Light: Stop, idle, and cool before topping off coolant.
🔋 Battery Light: Check terminals.
💨 EPS Light: Power steering overheated — take a break.

💡 TTN Tip: Take a photo of your dash when it’s “normal.” It’s your baseline for troubleshooting later.

### 🛞 Step 5 — Drive Modes & Terrain Control

Most SxS rigs have drive mode selectors for different conditions.

| Mode | Use | Behavior |
| --- | --- | --- |
| 2WD | Flat or hardpack | Rear tires only |
| 4WD (AWD) | Trails, hills | All tires engage automatically |
| 4WD Lock | Deep mud, climbing | All tires pull evenly |
| Turf Mode | Pavement, grass | Rear tires spin separately to protect surfaces |

💬 TTN Truth: If the steering suddenly feels stiff, you’re in 4WD. If it’s light and easy, you’re in 2WD.

🧰 Garage Lesson #6 — “Mode Match Drill”
1️⃣ Find open ground.
2️⃣ Switch between 2WD and 4WD several times while moving slowly.
3️⃣ Feel how the steering weight changes.
4️⃣ Note the difference in traction on loose vs. firm ground.

🧠 Goal: Understand how drive modes feel, not just what they do.

### 🚗 Step 6 — Your First Drive (Confidence Circuit)

Find a wide, flat, open space — no trees, no people, no pressure.

1️⃣ Start in Low Gear (L).
2️⃣ Ease off the brake until you roll.
3️⃣ Lightly touch throttle and drive a straight line 100 feet.
4️⃣ Stop gently.
5️⃣ Reverse 20 feet, looking behind you.
6️⃣ Make slow figure 8s both directions.
7️⃣ Practice stopping smoothly — no jerks.

💡 TTN Tip: Pretend there’s a cup of coffee on your dash. Smooth throttle and brake mean no spills — that’s the goal.

💬 TTN Truth: Every pro started by learning to move without drama.

### ⛰️ Step 7 — Basic Hill Practice

Uphill:

- Use Low Gear.

- Keep steady throttle, lean forward.

- Don’t stop mid-hill unless necessary.

Downhill:

- Stay in Low Gear for engine braking.

- Apply light brake pressure only.

- Never coast in Neutral or Park.

🧰 Garage Lesson #7 — “Mini Hill Drill”
1️⃣ Find a gentle slope.
2️⃣ Climb halfway, stop, reverse straight down.
3️⃣ Repeat three times smoothly.
🧠 Goal: Learn to trust gravity without panic.

💬 TTN Truth: Panic is just your body asking for a plan — and now you have one.

### 💨 Step 8 — Braking & Throttle Control

Throttle control separates new drivers from seasoned riders.
Smooth, patient movements = cooler belts, calmer rides, and longer machine life.

🧰 Garage Lesson #8 — “Smooth Start Drill”
1️⃣ Start in Low gear.
2️⃣ Roll 20 ft using slow, constant throttle.
3️⃣ Stop gently.
4️⃣ Repeat, increasing distance and speed slightly.
💡 TTN Tip: The smoother you start, the more control you’ll have when things get bumpy.

### 🗣️ Step 9 — Steering Feel & Terrain Feedback

Power steering makes driving easier but can trick you into over-correcting.

- Keep hands at 9 and 3 o’clock.

- Don’t palm-spin — stay controlled.

- Let the wheel “dance” a little on rough ground — that’s feedback, not danger.

💬 TTN Truth: A good driver listens through their hands — not just their ears.

### 🧰 Garage Lesson #9 — “Figure-8 Control Drill”

1️⃣ Mark two cones or bottles 30 ft apart.
2️⃣ Drive slow figure 8s around them.
3️⃣ Focus on smooth throttle, even turns, no jerky motion.
4️⃣ Try it again faster only when the first set feels natural.

🧠 Goal: Build control and awareness through rhythm — precision over speed.

### ✅ Confidence Challenges

☑️ Buckle in correctly every time — without thinking.
☑️ Shift through all gears smoothly.
☑️ Drive forward and reverse confidently.
☑️ Maintain calm throttle and brake control.
☑️ Recognize all major dash indicators.

💡 TTN Tip: Film your first practice ride. One month later, film again — and watch how your confidence shows in your posture, not just your speed.