# 🩵 LEVEL 1 — THE NEW RIDER

## SECTION 3 — PRE-RIDE & MAINTENANCE BASICS

(TTN Garage-Class Edition — “Preparation Builds Confidence, Confidence Builds Fun”)

### 🧠 Step 1 — The Why Behind the Checks

Every confident rider has a ritual — and this is yours.
A pre-ride check isn’t about being paranoid. It’s about being prepared.
Machines don’t break suddenly — they warn you first. You just need to know how to listen.

💬 TTN Truth: You can’t control the trail — but you can control your prep.

When you take five minutes to inspect your SxS before every ride, you’re preventing hours of trail-side frustration, towing, or worse — a weekend-ending breakdown.

💡 TTN Tip: Think of your pre-ride like buckling your seatbelt — it’s not exciting, but it keeps you riding.

### 🧩 Step 2 — The TTN Pre-Ride System

We use the same checklist every ride — it’s easy to remember:

T-F-B-B-B-L-G
(Tires • Fluids • Belt • Bolts • Brakes • Lights • Gear)

Each one takes less than a minute once you’ve practiced a few times.

## 🛞 T — Tires

Why It Matters: Tires are your connection to the ground. Even one low or damaged tire changes how the whole machine handles.

Check For:
✅ Air pressure (usually 14–18 PSI; check your manual).
✅ Cuts, punctures, or nails in tread.
✅ Cracks around sidewalls.
✅ Even wear between all four.

Tools You’ll Need:

- Tire pressure gauge

- Portable air compressor or hand pump

- Soapy water in a spray bottle (for leak checks)

💡 TTN Tip: Spray soapy water over suspicious spots. If it bubbles, you’ve got a leak.

❌ If You Find This:

- One tire low: Check for leaks, air up, and recheck after 10 minutes.

- Two tires low on the same side: You might’ve hit a rock ledge — inspect wheels for dents.

- Slow leak but no bubble: Check valve stem — tighten gently with a core tool.

## ⛽ F — Fluids

Fluids are your SxS’s lifeblood. Each one has a purpose — and neglecting any of them shortens your machine’s life.

### 🛢️ 1️⃣ Engine Oil

Where It Is: Dipstick near the engine (rear or midsection).
What to Look For:

- Color: golden to light brown = healthy.

- Smell: burnt = overheating.

- Level: between “Low” and “Full.”
💡 TTN Tip: Always check oil when the machine is cold and level.

❌ If You Find This:

- Milky oil: Coolant leak — don’t ride.

- Low oil: Top off and recheck after your ride.

### 💧 2️⃣ Coolant

Where It Is: Plastic overflow tank near radiator (front or behind seats).
What to Look For:

- Between “Min” and “Max” lines.

- Clean, brightly colored fluid (usually green, orange, or pink).

❌ If You Find This:

- Low coolant: Top off with 50/50 premix coolant.

- Brown sludge: Flush system — corrosion buildup.

### 🛞 3️⃣ Brake Fluid

Where It Is: Small clear reservoir near driver-side firewall or under front hood.
What to Look For:

- Light golden color.

- Level near “Full.”

💬 TTN Truth: Brake fluid absorbs moisture — if it looks dark, replace it.

### ⚙️ 4️⃣ Differential & Transmission Fluid

Where It Is: Small fill plugs near front/rear axles and transmission case.
Check monthly or every 25–50 hours.

💡 TTN Tip: A drip of clear, clean oil is good. Milky = water intrusion.

## 🌀 B — Belt

Your drive belt connects the engine to the tires — it’s the most critical wear item on your SxS.

Where It Is: Inside the clutch housing behind the passenger rear tire (Polaris) or under side panel (Can-Am).

Check For:
✅ Belt dust buildup (wipe clean with dry rag).
✅ Cracks, fraying, or shiny glazing on edges.
✅ Loose clutch cover bolts.

Tools You’ll Need:

- T40 or T45 Torx socket

- Belt removal tool (brand-specific)

- Flashlight

💬 TTN Truth: Belts don’t fail randomly — they fail from heat, dirt, or bad habits.

💡 TTN Tip: If your belt smells like burnt rubber, you were slipping it — use Low gear next time.

❌ If You Find This:

- Belt dust everywhere: Clean and recheck alignment.

- Cracked edges: Replace belt before it lets go on trail.

- Shiny ribs: Overheating — check clutch venting.

## 🔩 B — Bolts & Fasteners

A loose bolt can ruin a ride faster than a flat tire.
Focus on suspension, steering, and cage mounts.

Where To Check:
✅ A-Arms & radius rods
✅ Shock bolts
✅ Wheel lugs (100 ft-lbs for most SxS)
✅ Steering rack mounts
✅ Roof and accessory mounts

Tools You’ll Need:

- Torque wrench (10–100 ft-lbs range)

- Socket set

- Blue Loctite (medium strength)

🧰 Garage Lesson #10 — “Wiggle Test”
1️⃣ Grab each A-arm and pull side to side.
2️⃣ If it clunks or shifts, re-torque or inspect bushings.

💡 TTN Tip: Blue Loctite = removable. Red Loctite = permanent. Choose wisely.

## 🛑 B — Brakes

Brakes keep you alive on hills. Never assume they’re fine — confirm it.

Check:
✅ Pedal feels firm, not spongy.
✅ Brake pads visible with at least 1/8” material.
✅ No squealing or grinding.

❌ If You Find This:

- Soft pedal: Air or fluid leak.

- Squeak: Pads wearing out — time to replace.

- Grinding: Metal-on-metal — don’t ride.

💬 TTN Truth: Soft brakes are louder than any exhaust — they just whisper until it’s too late.

## 💡 L — Lights

Check headlights, taillights, whip lights, and brake lights before every ride.

Tools You’ll Need:

- Flashlight

- Spare fuses

- Multimeter (optional)

💡 TTN Tip: Carry a small 10-amp fuse kit in your glovebox — they’re trail lifesavers.

## 🎒 G — Gear & Tools

Always ride with the basics — because once you need them, it’s already too late to wish you had them.

Pack List:
🧰 Tool roll (sockets, wrenches, pliers)
🪛 Tire plug kit
🪫 Portable air compressor
🔋 Jumper box or cables
⛽ Funnel and spare oil
🔦 Flashlight
🚑 First aid kit
📻 Radio or comms
🗺️ OnX or paper map

💬 TTN Truth: The best riders carry tools for others — not just themselves.

### 🧰 Garage Lesson #11 — “Hands-On 10-Minute Pre-Ride”

Set a timer for 10 minutes and run through this checklist:
1️⃣ Walk around your SxS — look for leaks, low tires, or loose parts.
2️⃣ Check fluids (oil, coolant, brake).
3️⃣ Pop clutch cover and inspect belt edges.
4️⃣ Shake suspension arms and steering.
5️⃣ Test lights and brakes.
6️⃣ Verify all switches work.

🧠 Goal: Make this a habit. The more you repeat it, the faster it becomes second nature.

### ❌ TTN Troubleshooting Quick-Guide

| Symptom | Likely Cause | Quick Fix |
| --- | --- | --- |
| Oil smell | Low oil or leak | Check dipstick & drain plug |
| Rattle at idle | Loose panel or skid plate | Tighten bolts |
| Squeak when turning | Dry bushing | Grease pivots |
| Whine while accelerating | Belt slipping | Clean clutch housing |
| Brake feels soft | Air or water in lines | Pump brakes & inspect fluid |

💡 TTN Tip: If something doesn’t feel right, stop and look. Curiosity saves belts, engines, and egos.

### 🧾 TTN 10-Minute Pre-Ride Card (Printable)

🛞 Tires — Check pressure & damage
⛽ Fluids — Oil, coolant, brake
🌀 Belt — Clean & uncracked
🔩 Bolts — Suspension & lugs tight
🛑 Brakes — Firm pedal, no squeak
💡 Lights — All working
🎒 Gear — Tools, first aid, comms packed

💬 TTN Truth: Confidence doesn’t start on the trail — it starts in the driveway.