CUSTOM CAGE BUILD – 2 SEATER

BUILD SHEET + TECH CHECKLIST + TIER BREAKDOWN

TIER BREAKDOWN

Tier 1 (Trail Series)

- DOM 1.75x.095 main tubes

- Basic roof halo + A/B pillars

- 1 roof crossbar

- 4 gussets

- Standard tabs

- Aluminum roof

- Powdercoat single color

Tier 2 (Adventure Series)

- DOM 1.75x.120 main tubes

- 2 roof crossbars

- 8 gussets

- Roof visor

- Whip tabs, light bar tabs

- Roof ribs

- Aluminum closeouts

- Upgraded powdercoat

Tier 3 (Elite Series)

- Full DOM 1.75x.120 + 1.5 support tubes

- 2 crossbars + X-brace

- 12 gussets

- Windshield bar

- Chase light tabs

- Roof rack mounts

- Reinforced roof ribs

- Premium powdercoat + texture options

TECHNICIAN CHECKLIST

Tubing

- Cut A pillars

- Cut B pillars

- Form halo

- Notch tubes

- Check symmetry

- Weld halo

- Fit A/B pillars

- Add crossbars

- Add harness bar

- Add intrusion bar

Brackets & Tabs

- Weld mounting plates

- Fit light bar tabs

- Fit whip tabs

- Fit chase bar tabs

- Fit roof panel tabs

Panels

- Fit aluminum roof

- Drill roof hardware

- Install isolators

- Close-out panels if needed

Finishing

- Prep welds

- Smooth edges

- Powdercoat prep

- QC inspection

SUPPLIES QUANTITIES

- Main halo: 1

- A-pillars: 2

- B-pillars: 2

- Crossbars: 2

- Rear down-bars: 2

- Harness bar: 1

- Dash/intrusion bar: 1

- Roof tabs: 8

- Gussets: 8–12

- Mounting plates: 4

- Light bar tabs: 2

- Whip tabs: 2

- Chase tabs: 2

- Aluminum roof: 1